Digital Control Architectures and Technologies (01PDCQW-01PDCOV)

Surname       Name      id

am Simulation

Design Report

Delivery of this filled out report is mandatory for evaluation of Part II

Develop the design procedure through a

- MatLab script named sxxxxxx_design.m following the naming described below for the relevant transfer functions and variables:

  - Gcont à continuous time plant transfer function

  - G à discrete time plant transfer function ;

  - C à discrete time cascade controller ;

  - Ts à sampling time ;

  - zeta à damping coefficient , wn à natural frequency ;

  - M_S à Sylvester matrix .

- Simulink file named sxxxxxx_sim.slx.

Save all the design results in the sxxxxxx.mat file using the statement 
save sxxxxxx  at the MatLab prompt  >> à >> save sxxxxxx

In all the above files, replace sxxxxxx with your own id.

Plant discretization (cut and paste from matlab the discretized the plant tf G(z) in zpk form)

G(z) =

0.00049669 (z+0.9934)

---------------------

(z-1.009) (z-0.9717)

Steady state requirements analysis (report here the preliminary shape of C(z), based on the given steady state requirements highlighting the needed number of pole at 1 l_2 and how you set additional conditions on steady state)

l_2=1 
C(z) = S(z)/((z-1)R’(z))

Transient requirements analysis (report here the initial values of zeta and wn based on the given transient requirements)

zeta = 0.4037

wn = 4.5577

Design settings (report here zero-pole cancellation settings based on polynomials A+, A-, B+ and B-, and the final shape of C(z))

A_plus = z-0.9717

A_minus = z-1.0087

B_plus = 1

B_minus = 4.9669e-04*(z0.9934)

C(z)= A_plus*S’/((z-1)R’)

Degree Conditions (report here the degree conditions and the poles to assign)

Deg(S’) = 2       Deg(R’) = 2     Deg(A_m) = 4

Controller design (Report here the settings of the design highlighting the shape of the Diophantine equation, how additional conditions are handled and the final Sylvester matrix)

p1c=-zeta*wn+1j*wn*sqrt(1-zeta^2);

p1=exp(p1c*Ts);

p2=conj(p1);

p3c=-5*zeta*wn;

p3=exp(p3c*Ts);

p4=p3;

z-1A-zR'z+B-zS'z=Am(z)

0.1*Ts5e-4p2+p1+p0-G1Aplus1q2+q1+q0=0

M_s =

    1.0000         0             0             0             0             0
   -2.0087    1.0000         0        0.0005         0             0
    1.0087   -2.0087    1.0000    0.0005    0.0005         0
         0        1.0087   -2.0087         0        0.0005     0.0005
         0             0        1.0087         0             0         0.0005
    1.0000    1.0000    1.0000    0.1132    0.1132     0.1132

Design refinement (Report here how you refine the design in case of need, e.g., by modifying zeta and/or wn to meet the given requirements)

Controller tf (cut and paste from matlab the final controller tf C(z) in zpk form)

C(z) =

5.3556 (z-0.9717) (z^2 - 1.984z + 0.9846)

-----------------------------------------

(z-1.001) (z-1) (z-0.8842)

Performance evaluation (report in the table below the obtained performance)

| Requirement | 1b | 1c | 2 | 3 |
| --- | --- | --- | --- | --- |
| Value | 5e-4 | 0 | 23% | 2.5 |

Cut and paste in the space below the plot of the significant simulation for variables y(t) and e(t) to provide evidence of all the requirements values reported in the table above

Save data   >> save sxxxxxx (replace sxxxxxx with your own id)

Files to be delivered, besides the present report (if missing, or in wrong format or not consistent each other, Part II is not evaluated)

sxxxxxx.mat

sxxxxxx_design.m

sxxxxxx_sim.slx